| Field | Position (size, type) | Description |
| --- | --- | --- |
| Instrument | 1+n (n,a) | First Instrument Identifier: - N : Navcam - Z : Mastcam-Z - L : Supercam RMI - O: Orbital Mesh - P : PIXL Micro Context Cam (MCC) - I : SHERLOC Imaging (WATSON) - C : SHERLOC ACI - F : Front Hazcam (RCE-A) - B : Front Hazcam (RCE-B) - R : Rear Hazcam - E : EDL Camera - H : Mars Helicopter Scout Cam - V : Mars Helicopter Navigation Cam |
| _ | 2+n (1,a) | Underscore for readability. Always set to “_”. |
| Eye | 3+n (1,a) | Identifies the eye of the data used to make this mesh. - L : Left-eye - R : Right-eye |
| Mesh Type | 4+n (1,a) | Identifies the type of mesh. - T : Tactical - C : Contextual - H : Helicopter - O : Other |
| Special | 5+n (1,a) | Special Processing flag. The special processing character is used to indicate off-nominal or special processing of the mesh. Examples include use of different correlation parameters, special stretches to eliminate shadows, reprocessing with different camera pointing, etc. The meaning of any individual character in this field (other than "_" which means nominal processing) will be defined on an ad-hoc basis as needed during the mission. Within one Sol or a range of Sols, the character will be used consistently. So, this field can be used to group together all derived products resulting from one kind of special processing. An attempt will be made to maintain consistency across different Sols as well, but this may not always be possible; thus the meaning of characters may change across different individual or ranges of Sols. An ASCII text file will be maintained containing all special processing designators that are used, the Sols they relate to, and a description of the special processing that was done. This file will be included in the PDS archive. |
| Sol | 6+n (4,i) | - Sol : for flight (Cruise & Surface) or flight-like ground tests when the Secondary timestamp continuously increments, this field stores the 4-integer Sol (Mars solar day) of the first (i.e lowest) clock time acquired instrument data. - Year : for ground tests when the Secondary timestamp resets and repeats, the uniqueness as a timestamp is lost. Wall Clock is used to restore uniqueness, represented in a UTC-like format across Primary, Secondary, and Tertiary time fields. This field is used to store the Earth Year portion of the UTC-like time value, designated by a “Y” followed by the last three digits of the year. Time Type; Values; Valid Time Range. Sol; Cruise; 0000; NULL. Surface; 0001 – 9999; 1 thru 9999. _ _ _ _; Value is out of range. Year of UTC; Cruise Test; Y000; NULL. Surface Test; Y017 – Y099; 2017 thru 2099. _ _ _ _; Value is out of range. |
| Multi sol | 10+n (1,a) | Flag indicating that a significant percentage of the data content was acquired on more than a single Sol, i.e. across multiple Sols. Specification of this flag is at the discretion of the mosaic product's producer, who determines what percentage is "significant". Valid values: - _ : Flags mosaic as single sol (insignificant multiple sol contribution) - X : Flags mosaic as containing significant content from multiple sols |
| XYZ Type | 11+n (3,a) | Indicates the XYZ type used for mesh data: - See Table 6.3.1.2 |
| Geometry | 14+n (1,a) | Linearization flag: - _ : Non-linearized (raw geometry) - L : Product has been linearized with nominal stereo partner - A : Product has been linearized with an actual stereo partner. Note that for the “A” case, an image can have multiple stereo partners and the linearized images will be different for each partner. A user will need to look in the ODL/VICAR label to determine which partner was used for linearization. |
| Frame | 15+n (1,a) | Coordinate system (frame) type. Valid values: - S : Site frame - L : Local Level frame - R : Rover Frame - O : Other |
| Resolution | 16+n (1,i/a) | Indicates if the input data is comprised of single or multi-resolution images (ECAM reconstructed). Number value indicates level of pixel averaging. - 1 : 1x1 - 2 : 2x2 - 4 : 4x4 - 8 : 8x8 - M : multi-resolution |
| Pyramid | 17+n (1,i) | Indicates the mesh pyramid level or downsampling relative to the highest resolution of the mesh. (powers of 2). - 0 : full resolution - 1 : 2x downsampling - 2 : 4x downsampling - 3 : 8x downsampling - 4 : 16x downsampling - n : 2^n downsampling |
| Venue | 18+n (1,a) | Mission venue identifier: - _ : Flight (surface or cruise) - A : AVSTB - F : FSWTB - M : MSTB - R : “ROASTT” - S : “Scarecrow” - V : VSTB Other venue identifiers may be defined later. |
| Texture Type | 19+n (3,a) | Indicates the product type used for the mesh skin: - See Table 6.3.1.2 - _ _ _: no mesh skin |
| Site | 22+n (3,i/a) | Site identifier (see section X.X on Site frames): Site location count from the RMC where the data was acquired. Values; Range. 000, 001, …, 999; 0 thru 999. A00, A01, …, A99; 1000 thru 1099. B00, B01, …, B99; 1100 thru 1199. …; …. Z00, Z01, … Z99; 3500 thru 3599. AA0, AA1, …, AA9; 3600 thru 3609. AB0, AB1, …, AB9; 3610 thru 3619. …; …. ZZ0, ZZ1, …, ZZ9; 10350 thru 10359. AAA, AAB, …, AAZ; 10360 thru 10385. ABA, ABB, …, ABZ; 10386 thru 10411. …; …. ZZA, ZZB, …, ZZZ; 27910 thru 27935. 0AA, 0AB, …, 0AZ; 27936 thru 27961. 0BA, 0BB, …, 0BZ; 27962 thru 27987. …; …. 7CA, 7CB, …, 7CZ; 32720 thru 32745. 7DA, 7DB, …, 7DV; 32746 thru 32767. _ _ _; Value out of range. |
| Multisite | 25+n (1,a) | Flag indicating that a significant percentage of the data content was acquired on more than a single Drive position, i.e. across multiple drives. Valid values: - _ : Flags mesh as single Site - X : Flags mesh as containing content from multiple Sites |
| Drive | 26+n (4, i/a) | Drive identifier: Drive count (position within a Site location) from the RMC where the data was acquired. Values; Range. 0000, 0001, …, 9999; 0 thru 9999. A000, A001, …, A999; 10000 thru 10999. B000, B001, …, B999; 11000 thru 11999. …; …. Z000, Z001, … Z999; 35000 thru 35999. AA00, AA01, …, AA99; 36000 thru 36099. AB00, AB01, …, AB99; 36100 thru 36199. …; …. AZ00, AZ01, …, AZ99; 38500 thru 38599. BA00, BA01, …, BA99; 38600 thru 38699. BB00, BB01, …, BB99; 38700 thru 38799. …; …. LJ00, LJ01, …, LJ35; 65500 thru 65535. _ _ _ _; Value is out of range. |
| Multidrive | 30+n (1,a) | Flag indicating that a significant percentage of the data content was acquired on more than a single Drive position, i.e. across multiple drives. Valid values: - _ : Flags mesh as single Drive position - X : Flags mesh as containing content from multiple Drive positions |
| Description | 31+n (7,a) | General purpose mesh identifier. Can be set to anything to help identify the mosaic, such as target name, panorama name, theme name, method of production, etc. Valid values include A-Z, 0-9, and underscore. Must always pad to 7 characters, using underscores as necessary. |
| Producer | 38+n (1,a) | Identifier for the institution/team that created this product: - J : JPL (IDS/MIPL) Other producer codes will be added in the future. |
| Version | 39+n (2,a) | Product version number. Increments by one whenever a previously generated file with an otherwise identical filename exists. Values; Range. 01, 02 …, 99; 1 thru 99. A0, A1, …, A9; 100 thru 109. AA, AB, …, AZ; 110 thru 135. B0, B1, B2 …, B9; 136 thru 145. BA, BB, …, BZ; 146 thru 171. …; …. Z0, Z1, …, Z9; 1000 thru 1009. ZA, ZB, …, ZZ; 1010 thru 1035. _ _; Value is out of range. - Every version need not exist. E.g. version 01, 02, and 04 may exist but not 03. In general, the highest-numbered version represents the best version of that product. This field increments independently of all fields. |
| . | 40+n (1,a) | Separator for filename and extension. Always set to “.” |
| Extension | 41+n (3,a) | File extension: - iv - ht - mod - mtl - obj |